- OPERATION

  - CONTROLS AND FUNCTIONS

- BASIC FRONT PANEL KCH-3

- POWER switch

Press to turn the power ON and OFF.

- MON (MONITOR) switch

If the MON switch is pressed once while the radio is waiting for reception, the

coded squelch is cancelled. If the switch is pressed for two seconds, the

noise squelch is disabled and the speaker audio unmutes.

- OPT (OPTION) switch

Can be programmed for and operate any of the following: the PA (public

Address), HA (horn alert), TA (talk-around), AN (alphanumeric), IC (intercom),

SP (external speaker), D/A (delete/add), OPT (option), or AUX (auxiliary)

function programmed by the FPU (KPG-7D).

- SCN (SCAN) switch

Pressing this switch toggles the system scan function.

- D/A (DELETE/ADD) switch

(See section 4-5 and 4-6 for additional operations.)

If you hold down the SCN key and press this key, the selected channel can

Be added to or deleted from the scan list (Non-priority channels).

- INTERNAL SPEAKER

Front mounted for best sound quality.

- CH (CHANNEL) selector

Rotary control knob for channel selection.

- VOL (VOLUME) control

Audio level control knob: for receive, PA, IC volume.

- 13 SEGMENT display

Displays operating channel number.

Alphanumeric data for each channel is displayed, if applicable.

-  (CHANNEL ADD) indicator

If the selected channel is in the scan list, the indicator lights at that channel.

- SCN (SCAN) indicator

Indicates when scan mode is enabled.

-  (DOT) indicator

Lights when the programmed AUX switch is pressed.

- P1 (PRIORITY 1) indicator

Lights when a PRIORITY 1 channel is shown on the display.

- P2 (PRIORITY 2) indicator

Lights when a PRIORITY 2 channel is shown on the display.

- TA (TALK AROUND) indicator

Lights when the transceiver is in talk around mode (TA switch).

- SP (EXTERNAL SPEAKER) indicator

Lights when the received signal is output to the PA speaker.

- OPT (OPTION) indicator

Lights when the programmed OPT switch is pressed.

- MON (MONITOR) indicator

Lights when the MON switch is pressed and the coded squelch is disabled.

19. CALL indicator (An optional SIGNALING unit is required: 2-tone or DTMF decoder)

Lights when called individually.

- BUSY indicator

Indicates when channel is in use.

- TX indicator

Indicates when the transmitter is keyed.

- FULL FEATURE FRONT PANEL

KCH-4

1.     POWER BUTTON

Press to turn the power ON and OFF.

- GRP (GROUP) UP switch

GROUP UP switch for group selection. Holding the switch causes the

function to repeat.

- GRP (GROUP) DOWN switch

Group down switch for group selection. Holding the switch causes the

function to repeat.

- MON (MONITOR) switch

If the MON switch is pressed once while the radio is waiting for reception, the

coded squelch is cancelled. If the switch is pressed for two seconds, the

noise squelch is disabled and speaker audio unmutes.

- SCN (SCAN) switch

Pressing this switch toggles the system scan function.

- KEY 1~4 switches

Can be programmed for and operate any of the following: PA (public

address), HA (horn alert), TA (talk around), IC (intercom), AN (alphanumeric),

SP (external speaker), D/A (delete/add), OPT (option) or AUX (auxiliary)

function (programmed) by the FPU (KPG-7D).

- CH (CHANNEL) selector

Rotary control knob for channel selection.

- VOL (VOLUME) control

Audio level control knob: for receive, PA, IC volume.

- TX indicator

Indicates when the transmitter is keyed.

- MON (MONITOR) indicator

Lights when the MON switch is pressed and the coded squelch is disabled.

- SCAN indicator

Indicates when scan mode is enabled.

12.	P1 (PRIORITY 1) indicator

Lights when a priority 1 channel is shown on the display.

- P2 (PRIORITY 2) indicator

Lights when a priority 2 channel is shown on the display.

- SP (EXTERNAL SPEAKER) indicator

Lights when the received signal is output to the PA speaker.

- OPT (OPTION) indicator

Lights when the programmed option switch is pressed.

- TA (TALK AROUND) indicator

Lights when the transceiver is in talk around mode (TA switch).

- AUX (AUXILLARY) indicator

Lights when the programmed AUX switch is pressed.

- 13 SEGMENT display

Displays operating group number and channel number (channel display), or

channel name (12 character Alphanumeric display) programmed by the FPU

(KPG-7D).

-  (CHANNEL ADD) indicator

If the selected channel is in the scan list, the indicator lights at that channel.

-  (GROUP ADD) indicator

If the selected group is in the scan list, the indicator lights at that group.

- BUSY indicator

Indicates when channel is in use.

- CALL indicator (An optional SIGNALLING unit is required: 2-tone, DTMF

decode).

Lights when called individually.

  - RECEIVE

- To turn on the radio.

- Press the power switch in the upper left corner of the control unit once.

The display and graphics illuminate to indicate the radio is on.

- When turned on, all the indicators light, a confirmation tone sounds for

About a second, and the transceiver checks itself.

- To set volume

- To unsquelch the radio, press the MON switch for 2 seconds. This

unmutes the speaker.

(2)    Adjust the volume by using the VOL control.

- To change channels

KCH-3

Turn the CH (channel) selector to your regular operating channel.

KCH-4

Turn the CH (channel) selector to your regular operating channel on the

currently displayed group.

- To change groups. (only KCH-4)

Select the group by pressing either the GROUP UP or GROUP DOWN

switch. When the group number is changed, the channel display indicates the

revert channel number (channel displayed when the group was last selected).

- To enter carrier squelch mode.

If you wish to monitor (carrier squelch) a channel, press the MON switch.

You may also enter the monitor mode by taking the microphone off hook.

This function can be turned on or off by the FPU.

    - TRANSMIT

First, lift the microphone from its hook and listen for a few seconds to make

certain the channel is not being used.

- Normal On Hook Scan mode:

If scanning, the scanning stops temporarily, and the off hook revert channel

is displayed.

- On Hook Scan mode:

If scanning, pressing the PTT switch will stop the radio on the off-hook revert

channel and begin transmission immediately. See section 4-4. SCAN

OPERATION.

Now, hold the microphone about one and one-half inches from your lips, press

The PTT (Push-To-Talk) switch, and speak in your normal speaking voice. The

TX display will light during transmission. When you are through speaking,

Release the PTT switch, and you are automatically back in the receive mode.

When the conversation is finished, place the microphone back on its hook.

4-4.	SCAN OPERATION

    - General

There are two “mic hook” scan modes in which the radio can be programmed

to operate.

• 	On Hook Scan:

Normal method of scanning that requires the microphone to be on hook (in

the mic hanger) to initiate scan, and for channel scanning to occur.

•	Off Hook Scan:

The microphone does not have to be on hook to initiate scan or to scan

channels. Off hook scan should be used on radios implementing a

microphone headset or handset that use a combined PTT hook switch.

(1)	The scan feature is enabled by pressing the front panel SCN switch.

(2)	A single confirmation tone sounds and scanning starts. If there is one or

less added channels, an error tone sounds, and scanning does not start.

(3)	The display shows SCAN (or SC) and the SCAN (SCN) indicator lights.

(4)	If the transceiver receives a signal while scanning, the scanning stops

temporarily, the audio is unmuted, and the channel is displayed.

1. If the channel is not the priority channel and there is a priority channel,

the radio receives the signal alternately between the channel and the

priority channel.

- If the channel is the priority 2 channel and there is a priority 1 channel,

the radio receives the signal alternately between the priority 2 and the

priority 1 channel.

- If there is no priority channel, the scanning stops at that channel and

receives only that channel.

(5)	If the SCN switch is pressed again, an operation confirmation tone sounds,

and the scanning  ends (scan mode is exited). (The channel selected

immediately before the scanning began returns.)

    - To transmit while scanning.

# On Hook Scan

(1) 	Take the microphone off hook. The radio goes to the off-hook revert

channel.

(2)	When the PTT switch is pressed, transmission takes place on the displayed revert        channel.

Off Hook Scan

Upon pressing PTT, the radio ceased scanning and goes to the off-hook

revert channel on which transmission begins immediately.

    - To delete nuisance channels while scan is on.

(1)	To temporarily delete undesirable channels, press the D/A switch 2 seconds while the channel
 is active (indicated by the display).

A confirmation tone sounds, the currently displayed channel is deleted, and

The scanning resumes. The last channel in the scan sequence cannot be

deleted. (the priority channel can only be deleted if the “priority temporarily delete” feature is enabled
in the “scan information” programming.)

- To restore the original scan list, either turn scan off and on or turn the radio off and on.

    - To enter carrier squelch scan while scan is on.

If you have pressed the MON switch momentarily, MON indicator is turned on

and the radio operates carrier squelch scan.

4-5.	ADDING CHANNELS AND GROUPS TO THE SCAN LIST

(1)	Non-priority channel (A switch is programmed for D/A)

To add a channel to the scan list with the CHANNEL ADD indicator off, hold down the SCN switch, and press the D/A switch. The CHANNEL ADD indicator () lights and the channel is added to the scan list.

(2)	Second-priority (P2) channel (P2 = Fixed-operator selectable)

To add the currently displayed channel to the scan list as a priority 2

channel on a radio in which the second priority is operator selectable, hold

down the SCN switch and press the MON switch twice.

The CHANNEL ADD () indicator and P2 indicator light and the channel is

added to the scan list as P2.

(3)	First-priority (P1) channel (P1=Fixed operator selectable)

To add the currently displayed channel to the scan list as a priority 1 channel

on  a radio in which the first priority in operator selectable, hold down the

SCN switch and press the MON switch three times.

The CHANNEL ADD indicator () and P1 indicator light, and the channel is

added to the scan list as P1.

(4)	Groups (KCH-4: Full featured front panel / Group lock out = operator

selectable / Group scan = multi)

To add the currently displayed group of channels to the scan list, hold down

The SCN switch and press the GROUP UP switch. The GROUP ADD

Indicator () lights, and the group is added to the scan list.

4-6.	DELETING CHANNELS AND GROUPS FROM THE SCAN LIST

(1)	Non-priority channel (A switch is programmed for D/A)

To delete a channel from the scan list with the CHANNEL ADD ()

indicator on, hold down the SCN switch, and press the D/A switch. The

CHANNEL ADD indicator () goes off and the channel is deleted from the

scan list.

(2)	Second-priority (P2) channel (P2 = Fixed-operator selectable)

To delete the currently displayed channel from the scan list as a priority 2

channel on a radio in which the second priority is operator selectable, hold

down the SCN switch and press the D/A switch twice.

The P2 indicator goes off. Then hold down the SCN switch, and press the

D/A switch. The CHANNEL ADD indicator () goes off and the channel

becomes a non-priority channel.

(3)	First-priority (P1) channel (P1 = Fixed-operator selectable)

To delete the currently displayed priority 1 channel from the scan list on a

radio in which the first priority is operator selectable, hold down the SCN

switch and press the D/A switch three times.

The P1 indicator goes off and the channel becomes a non-priority channel.

Then hold down the SCN switch, and press the D/A switch. The CHANNEL

ADD indicator () goes off.

(4)	Groups (KCH-4: Full featured front panel / Group lock out = operator

selectable / Group scan = multi)

To delete the currently displayed group of channels from the scan list, hold

down the SCN switch and press the GROUP DOWN switch. The GROUP

ADD indicator () goes off, and all the channels in that group are deleted

from the scan list.

4-7.	REPEATER TALK-AROUND (TA)

TALK-AROUND is useful when you are close to the other mobiles with whom you

wish to talk, or are outside the range of your repeater system.

If you are using the repeater, press the TA switch momentarily. A confirmation

tone sounds, and the TA indicator lights. The transceiver can then transmit

without using the repeater.

To use the repeater, press the TA switch momentarily again. A confirmation

tone sounds, and the TA indicator goes off.

  - HORN ALERT FUNCTION (HA)

An option for calling is required to use this function.

If you are called from the base station while you are away from your transceiver,

you are alerted by the vehicle horn or some other type of external alert. To turn

the horn alert function on when you go away from your transceiver, press the

HA switch momentarily. A confirmation tone sounds and the display shows

HORN ALERT (or HA).

If the HA Switch is pressed again, horn alert function is turned off.

  - PUBLIC ADDRESS FUNCTION (PA)

Public Address amplifies the microphone audio and outputs it through a

dedicated PA speaker. The 20 W (maximum rating) speaker is usually a normal

cab type speaker mounted inside a large vehicle such as a bus, or a weather-

proof speaker mounted external to the vehicle.

PA mode is activated by pressing the front panel PA key (or other designated

switch). A confirmation tone sounds, and the display shows PUBLIC ADRS (or

PA). PA can be activated at anytime (scanning or non-scanning). The radio

continues to scan (if enabled) & receive calls while in PA mode. Pressing PTT

activates PA and will override and incoming call at anytime; however, no radio

transmission takes place. If PA is pressed again, a confirmation tone sounds,

the display returns to the normal channel or SCAN (SC) display, and the PA

function is defeated.

  - EXTERNAL SPEAKER FUNCTIONN (SP)

SP amplifies the received audio from the radio and outputs it through a

dedicated PA speaker. The 20 W (maximum rating) speaker is usually a normal

cab-type speaker mounted inside a large vehicle such as a bus, or weather-

proof speaker mounted external to the vehicle. The external speaker enables

the vehicle operator to hear incoming calls while outside the vehicle, but still

within hearing range of the speaker.

SP mode is activated by pressing the front panel SP key (or other designated

switch). A confirmation tone sounds and the display shows an SP icon. SP

can be activated at anytime (scanning or non-scanning). The radio transmits

and operates normally while SP is activated, but all received calls will be output

through the PA speaker. If SP is pressed again, a confirmation tone sounds,

the SP icon goes off and the SP function is defeated.

  - ALPHANUMERIC FUNCTION (AN)

This function switches the 13- segment display between alphanumeric display

and channel display.

If you want to change from alphanumeric display to channel display, press the

AN switch momentarily. A confirmation tone sounds and the alphanumeric

display changes to the channel display.

If the AN switch is pressed momentarily again, a confirmation tone sounds and

the channel display changes back to the alphanumeric display. (Requires

optional KCH-5 for alphanumeric on the Basic front (KCH-3).)

  - INTERCOM FUNCTION (IC) (Dual Control Head radios only)

Intercom (IC) allows one control head operator to talk to the other control head

operator such as in an ambulance or on a firetruck. IC mode is activated by

pressing the front panel IC key (or other designated switch). A confirmation

tone sounds and the display shows INTERCOM (or IC). IC can be activated at

anytime (scanning or non-scanning). The radio continues to scan & receive

calls while in IC mode. Pressing PTT activates IC and will override an incoming

call at anytime, however, no radio transmission takes place. If IC is pressed

again, a confirmation tone sounds, the display returns to the normal channel or

SCAN (SC) display, and the IC function is deactivated.

  - HOME CHANNEL (HC)

This feature allows the radio operator to immediately select a pre-determined

“home channel” by pressing the designated HOME CHANNEL key (as opposed

to selecting it by the channel knob and/or group keys). This feature operates in

both scanning and non-scan modes. If your dealer has assigned the HOME

CHANNEL to a front panel key, it can be selected as follows:

Home Channel access:

- Press the HC key once.

Non-Scan Mode: The display will change to the home channel.

Scanning Mode:	The transceiver will momentarily stop scan and display

The home channel, then scanning will resume normally.

This channel will also become the new off-hook revert

channel.

To Select a “new” Home Channel (radio must be in non-scan mode):

- Select the desired channel with the channel/group selectors.

- Press and hold the SCN key while hitting HC.

- A single confirmation tone sounds; the new home channel is now set.

4-14.	OPERATOR SELECTABLE SQUELCH CONTROL (SQL)

This feature allows the mobile operator to manually adjust the internal squelch

threshold in 16 steps via the front panel controls. For carrier squelched

channels, an occasion my arise where the current setting is too sensitive for

the current RF environment that the transceiver is located, thus causing the

squelch to open and close intermittently. This feature can only be initiated in

non-scan mode. If your dealer has assigned the OPERATOR SELECTABLE

SQUELCH to a front panel key, it operates as follows:

- Initiate the squelch setting mode.

  - KCH-4 fronts: Press the SQL key once

  - KCH-3 fronts: Press the key that has been assigned the “SQL” function.

This is either the SCN, MON, OPT or D/A key. The display will change

to show the current squelch level setting. (Default = Code 9):

Examples:

KCH-4 Front Panel		KCH-3 Front Panel

Code 9			‘9

Selectable range: Code 1 to code 16.

- Select a new squelch level.

- Rotate the channel select knob. The LCD will increment to the next

highest (clockwise) or lowest (counterclockwise) setting as the knob is

rotated. It is recommended that the setting be increased by increments

of one step level at a time, and then tested to see if this setting is

acceptable. The transceiver will continue to receive calls on the last

selected channel while in this mode.

- Exit the squelch select mode.

- Press the SQL key one. The LCD will return to the previous channel

display. The new setting is memorized in eeprom and will remain until

changed by the operator again.

Note: for dual band transceivers, the squelch setting can be done for

both frequency bands by selecting a channel in each band, and

performing procedures (1) through (3) above on each channel.

END